KANAWHA COUNTY COMMISSION

P. O. BOX 3627

407 VIRGINIA STREET, EAST

CHARLESTON, WEST VIRGINIA 25336

(304) 357-0117

Request for Bids

Re:	Request for Bids for Two (2) VHF Simplex Stations for the Kanawha County Commission

Date:	October 10, 2003

]

Fiscal Year:	2003-04

Bid Opening:	Bids must be received on or before Tuesday, October 28, 2003, at 2:00 p.m. in the Kanawha County Commission Purchasing Office, 407 Virginia Street, East, Third Floor, Room 229, Charleston, WV 25301 (P.O. Box 3627, Charleston, WV 25336)

INSTRUCTIONS TO BIDDERS:

*PLEASE USE THIS FORM AS THE COVER SHEET FOR YOUR BID

1.	Bids must be received in a sealed envelope with the date and time of the bid opening on the outside of the envelope.  Faxed bids will not be accepted.

2.	Bid must be F.O.B. Delivery Point, unless otherwise indicated in proposal.

3.	All bids should be signed and in ink, showing all facts and the total amount of the bid.

4.	The County reserves the right to accept or reject in part or in whole any bid submitted, whichever is in the best interest of the County.

| Item No. | Description | Bid Amount |
| --- | --- | --- |
| 1 | Two (2) VHF Simplex Stations per the Attached Specifications | $ |

Written Bid Amount : ________________________________________________________________________________________________________________________________________________________Dollars

DELIVERY DATE: ________________________

Vendor Name:  _____________________________     Signature: 	______________________

Address:           _____________________________     Date:  	______________________

_____________________________      Telephone:   _______________________

***ATTENTION VENDORS***

Have you registered for the Kanawha County Commission Automatic Vendor E-Mail Notification System for bid specifications?  Register by visiting our website @  www.kanawhacounty.com/purchasing/alerts .

# SPECIFICATIONS

COUNTY COMMISSION OF KANAWHA COUNTY

CHARLESTON, WEST VIRGINIA

ITEM:	Request for Bids for Two (2) VHF Simplex Stations

LOCATION:	Kanawha County Commission

409 Virginia Street, East

Charleston, WV 25301

CONTACT:		Jerie Whitehead			Steven P.J. Johnson

Purchasing Director			Chairman

Kanawha County Commission	Kanawha County Firefighters Mutual Aid

407 Virginia St., East		   	   Association Communications Committee

P.O. Box 3627				P.O. Box 2342

Charleston, WV 25336		Charleston, WV 25328

Telephone (304) 357-0115		(304) 949-2332

BID OPENING:	Bids must be received in a sealed envelope, with the date and time of the bid opening on the outside of the envelope, on or before Tuesday, October 28, 2003, at 2:00 p.m., in the Kanawha County Commission Purchasing Office, 407 Virginia Street, East, Third Floor, Room 229, Charleston, West Virginia 25301 (P.O. Box 3627, Charleston, WV 25336).  Faxed bids will not be accepted.

SPECIFICATIONS:   The following specifications are intended to describe two (2) VHF Simplex Stations for the Kanawha County Commission and the details contained in these specifications are not designed to exclude any vendor from bidding, but are offered as a means of describing the needs of the Kanawha County Commission.   All specifications are minimum requirements.

Phase Two of OES Radio Upgrade for Fire One:

Two (2) VHF Simplex Stations

The stations will be used to replace/upgrade existing stations located at the following sites:

One (1) station will replace the OES Western Fire Base located at Cross Lanes on a 300= ft structure owned and operated by Electronic Communications of W.VA Inc.

One (1) station will replace the OES Northern Fire Base located off of I-79 and off of Buff lick Road on a 300= ft structure owned and operated by Electronic Communications of W VA Inc.

The OES Radio Committee prefers that the above listed sites be used. However the bidders are advised that these sites are restricted. Kanawha County OES has an agreement with the owner of these sites stating that any vendor may utilize these bonded sites. In order to utilize these sites all installation and future maintenance will have to be performed by:

Electronic Communications of W. VA

408 Old Goff Mountain Road

Cross Lanes WV 25313

Attn: Tim Ford

Tel: 304-776-4091

The stations to be supplied shall be a tone controlled, one (1) frequency VHF station equipped with one (1) receiver, transmission limiting timer, EIA CTCSS tone squelch on the transmitter and receiver, and shielding for operation in a congested RF environment. The station shall be of EIA continuous duty cycle type and be of an all solid-state design. The use of fans or blowers is permitted.

The stations must be factory manufactured by a reputable communications company (i.e. Motorola, Kenwood, Macom, E.F. Johnson, or other) the unit must have a from the factory warranty with minimum of two years upon delivery.

The stations must be a model currently in production by the manufacturer.

The stations to be supplied shall operate on the frequency 155.145.

The stations shall be supplied in sturdy metal cabinets with doors and locks. No circuits shall be exposed with the doors in place. The stations shall be supplied with a Battery Backup system to last the minimum of 12 hours in a loss of power situation.  The transmitters shall meet the following minimum technical specifications:

Power 		120   VAC / 13.8 VDC

RF Power Output		100> watts

Frequency Stability		+ 0.00015%  ( -30 degrees to +60 degrees C )

Spurious & Harmonic		-85 dB

Emissions

Sideband Noise		-70 dB

FM Hum and Noise		-55 dB (16K0F3E)  - 50 (11K0F3E)

Audio Distortion 		< .05% max at 1000 Hz

Line Input Sensitivity		-20 to +11 dBm for 100% Modulation

Transmission Limiting Timer		off/ 0-30 min. To be adjusted to 3 minutes.

The receivers shall meet the following minimum technical specifications:

Sensitivity		0.30 or better uV 12 dB SINAD

0.40 or better uV 20 dB Quieting

Selectivity (EIA) @ 30KHz		-95 = > dB

@ 12.5 KHz		-87 = > dB

Intermodulation Rejection		-90 = > Db @ 30 KHz

-85 = > dB @ 15 KHz NBFM

Frequency Stability		+0.00015% (-30degrees to +60 degrees C)

Spurious & Image Rejection		-100 = > dB

FM Hum and Noise		-60 dB Wide / -55 Narrow

Audio Output (Ext Speaker)		4 watts> Adjustable

Audio Distortion (Ext Speaker)	< 2% maximum at 1 KHz

The receivers shall be configured in such a manner that it can be used as a voting receiver.

Two Motorola UHF Repeaters Model GR 1225 or equivalent shall be installed on existing UHF frequencies (1 Northern 1 Western).

Three AC Circuit Protectors 125VAC, 20Amp 3-wire plug, 800 joules such as a PolyPhaser ISPLDO12020A shall be provided. (To be installed at the following sites Western, Northern, and Eastern sites)

Five Lightning Protector, Bulkhead Mount, 50-700 MHz also rated for EMP, VHF 200 Watts, UHF 100 Watts, 2 female N connectors.

Such as PolyPhaser IS-NEMP-C1 shall be provided to be installed at each site.

Five-2 inch Super Flex Jumpers, length to be as such to go from RX/TX equipment     (1- VHF 1- UHF per site) to ground bar where Bulkhead Mount Lightning Protector is located all jumpers are to be assembled on site.

Five Service Manuals for all of the equipment shall be provided with all schematics and modifications detailed upon the completion of the install.

Five sets of keys for each station shall be provided upon installation.

All installation of the above equipment must be preformed by a FCC licensed Technician with a report to be given upon completion of the entire install.

Specification sheets on all equipment is to be provided with the bid package.

REQUIREMENTS:

1.	Include shipping charges

2.	2.	State warranty terms and conditions

3.	3.	State delivery date

Real Estate and Personal Property Taxes: ANo bid will be accepted or opened on any County

contract if the vendor is listed on the last published list of delinquent real or personal property taxes in Kanawha County; however,  the Commission will accept bids by vendors who provide satisfactory proof of payment of current taxes or a certification from the Sheriff that no taxes are due prior to submission of said bid.@

Required Forms:     Vendor shall complete and submit, or have on file with the County, a Vendor Registration and Disclosure Statement Form.

Rejection of Bids:  The Kanawha County Commission reserves the right to reject any and/or all

bids and to waive any informalities in bidding.