KANAWHA COUNTY COMMISSION

P. O. BOX 3627

407 VIRGINIA STREET, EAST

CHARLESTON, WEST VIRGINIA 25336

(304) 357-0117

Addendum #1

Re:		Request for Proposals for Voice Activated Audio/Video Systems and Presentation Systems for the 13th Judicial Circuit Court

Date:		October 1, 2003

Fiscal Year:	2003-04

Bid Opening:      Bids must be received on or before Tuesday, October 14, 2003, at 2:00 p.m. in the Kanawha County Commission Purchasing Office, 407 Virginia Street, East, Third Floor, Room 229, Charleston, WV 25301 (P.O. Box 3627, Charleston, WV 25336)

INSTRUCTIONS TO BIDDERS:

*PLEASE USE THIS FORM AS THE COVER SHEET FOR YOUR BID

1.	Bids must be received in a sealed envelope with the date and time of the bid opening on the outside of the envelope.  Faxed bids will not be accepted.

2.	Bid must be F.O.B. Delivery Point, unless otherwise indicated in proposal.

3.	All bids should be signed in ink, showing all facts and the total amount of the bid.

4.	The County reserves the right to accept or reject in part or in whole any bid submitted, whichever is in the best interest of the County.

| ITEM | DESCRIPTION | # ONE | # SEVEN |
| --- | --- | --- | --- |
| 1 | Audio/Video Courtroom Electronic System(s) per the Specifications Dated October 1, 2003, and Addendum #1 Dated October 10, 2003 | $ | $ |
| 2 | Motorized Projection Screen per the Specifications Dated October 1, 2003, and Addendum #1 Dated October 10, 2003 | $ | $ |
| 3 | Ceiling Mounted 3000 Lumen Projector per the Specifications Dated October 1, 2003, and Addendum #1 Dated October 10, 2003 | $ | $ |
| 4 | Evidence Presentation Cart & Equipment per the Specifications Dated October 1, 2003, and Addendum #1 Dated October 10, 2003 | $ | $ |
| 5 | First Year Warranty (One-Day Service) | $ | $ |
| 6 | Optional Second Year Warranty (One-Day Service) | $ | $ |
| 7 | Optional Third Year Warranty (One- Day Service) | $ | $ |

Vendor Name:  _____________________________     		Signature: ___________________________________

Address:           _____________________________     		Date:  	    ____________________________________

_____________________________  		Telephone:________________________________

# ADDENDUM # 1 TO SPECIFICATIONS

COUNTY COMMISSION OF KANAWHA COUNTY

CHARLESTON, WEST VIRGINIA

ITEM:			Request for Proposals for Voice Activated Audio/Video Systems and Presentation Systems for the 13th Judicial Circuit Court

LOCATION:		Kanawha County Commission

407 Virginia Street, East, Third Floor

Charleston, WV 25301

CONTACT:		Jerie Whitehead			Michael M. Phillips, AIA

Purchasing Director			Buchart Horn, Inc.

Kanawha County Commission	319 W. Washington Street

407 Virginia St., East		Charleston, WV 25302

P.O. Box 3627			Telephone (304) 346-1176

Charleston, WV 25336

Telephone (304) 357-0115

Jimmy Thaxton

Case Manager

Court Administrator’s Office, 13th Judicial Circuit Court

111 Court Street, Charleston, WV  25301

(304) 357-0388

BID OPENING:	Bids must be received in a sealed envelope, with the date and time of the bid opening on the outside of the envelope, on or before Tuesday, October 14, 2003, at 2:00 p.m., in the Kanawha County Commission Purchasing Office, 407 Virginia Street, East, Third Floor, Room 229, Charleston, West Virginia 25301 (P.O. Box 3627, Charleston, WV 25336).  Faxed bids will not be accepted.

Addendum #1:

Please be advised that our bid specifications have been altered to remove any requirement for videoconferencing equipment. Please remove any videoconferencing equipment or systems requests from your bid, specifically, the requirement that your bid include videoconferencing equipment capable of conducting videoconferences over Wide Area Networks, or that is capable of conducting video arraignments from the Regional Jails.

Specifically, the following items are no longer part of the requested bid, and should be considered stricken from the request.

Section 1. Purpose

"...Video conferencing equipment for each Courtroom must include a Codek (sic), controller, and other necessary peripheral equipment (spec sheet is to be included with bid).”

Subsection I. Audio/Video Portion of Courtroom Systems

"6. Have complete video arraignment and video conferencing interface and integration."

"16. System is to incorporate and facilitate videoconferencing from a regional jail, penitentiary or other remote location. To be a true two-way communication with the far end of the conference to be able to observe switched video and audio from the courtroom, but would not see himself or herself on the monitor at the far end. The court would be able to see the person constantly on the selected monitors in the courtroom. The audio is to be amplified into the courtroom as the person speaks. The video is to show from the court as well as the far end of the teleconference as well as the audio. The operation of this feature is to include a single switch on the system administrator’s bench, which would enable operation from the far end. Until the administrator activates the switch the far end monitor would only see himself or herself and would receive no video or audio from the courtroom. System must allow for a different operational procedure if desired."

"24. Video conferencing equipment for each Courtroom must include a Codek (sic), controller, and other necessary peripheral equipment (spec sheet is to be included with bid)."

Subsection A. The audio/video mixer is required to contain the following:

"Quad split capability in Videoconference mode"

Subsection IV. Electronic Evidence Presentation System

"6. Courtroom systems must integrate and interface with the videoconferencing in each Courtroom as part of the system. In this context, videoconferencing means using video technology to receive live testimony from a witness located off-site and to permit examination of the witness by the judge and the attorneys in the courtroom. At the same time, the witness shall be able to see and hear the person questioning him or her by means of the automatic switching of both microphones and video cameras to the person in the courtroom who is speaking."

"7. Video Conference systems need to use the H.323 protocol for LAN-based conference calls.  Preference will be given to proposals that can also support H.320 conversations."

Note again that all the above language is to be stricken from our request dated October 1, 2003, and that all other references referring to videoconferencing equipment or capability are hereby inoperative.

All Requirements, Terms & Conditions and Attachments to the Specifications dated October 1, 2003,,which are not addressed in this Addendum are to be included as a part of this Addendum #1.

The Kanawha County Commission reserves the right to reject any and/or all bids and to waive any informalities in bidding.