KANAWHA COUNTY COMMISSION

P. O. BOX 3627

407 VIRGINIA STREET, EAST

CHARLESTON, WEST VIRGINIA 25336

(304) 357-0117

Request for Bids

Re:			Walk-In Van for the Kanawha County Sheriff’s Department

Date:			November 13, 2003

Fiscal Year:		2003-04

Bid Opening:	Bids must be received on or before Tuesday, December 9, 2003, at 2:00 p.m. in the Kanawha County Commission Purchasing Office, 407 Virginia Street, East, Third Floor, Room 229, Charleston, WV 25301 (P.O. Box 3627, Charleston, WV 25336)

INSTRUCTIONS TO BIDDERS:

*PLEASE USE THIS FORM AS THE COVER SHEET FOR YOUR BID

1.	Bids must be received in a sealed envelope with the date and time of the bid opening on the outside of the envelope.  Faxed bids will not be accepted.

2.	Bids are to be signed.

3.	Bid must be F.O.B. Delivery Point, unless otherwise indicated in proposal.

4.	All bids should be in ink, showing all facts and the total amount of the bid.

5.	The County reserves the right to accept or reject in part or in whole any bid submitted, whichever is in the best interest of the County.

| Item No. | Description | Bid Amount |
| --- | --- | --- |
| 1 | Walk-In Van for the Kanawha County Sheriff’s Department per the attached Specifications | $ |

Written Bid Amount : ________________________________________________________________________________________________________________________________________________________Dollars

Vendor Name:  _____________________________     	Signature: _______________________

Address:           _____________________________     	Date:  	__________________________

_____________________________  		Telephone:   ________________________

***ATTENTION VENDORS***

Have you registered for the Kanawha County Commission Automatic Vendor E-Mail Notification System for bid specifications?  Register by visiting our website @                                                                                                                   www.kanawhacounty.com/purchasing/alerts .

SPECIFICATIONS

COUNTY COMMISSION OF KANAWHA COUNTY

CHARLESTON, WEST VIRGINIA

ITEM:	Walk-In Van for the Kanawha County Sheriff’s Department

LOCATION:	Kanawha County Commission

409 Virginia Street, East

Charleston, WV 25301

CONTACT:		Jerie Whitehead

Purchasing Director

Kanawha County Commission

407 Virginia St., East

P.O. Box 3627

Charleston, WV 25336

Telephone (304) 357-0115

BID OPENING:	Bids must be received in a sealed envelope, with the date and time of the bid opening on the outside of the envelope, on or before Tuesday, December 9, 2003, at 2:00 p.m., in the Kanawha County Commission Purchasing Office, 407 Virginia Street, East, Third Floor, Room 229, Charleston, West Virginia 25301 (P.O. Box 3627, Charleston, WV 25336).  Faxed bids will not be accepted.

SPECIFICATIONS:	The following specifications are intended to describe a walk-in van for the Kanawha County Sheriff’s Department and the details contained in these specifications are not designed to exclude any vendor from bidding, but are offered as a means of describing the needs of the Kanawha County Sheriff’s Department.  Where brand names may be used, the words Aor equal@ are assumed to follow.  All specifications are minimum requirements.

1.	Route Star Walk-in Van Model #WP31442 or equivalent

2.	157" wheelbase 16' body size

3.	14,100 lbs. GVWR

4.	192" load space length

5.	106"-110" overall height

6.	Diesel engine (HP4) Cummins ISB4 4-cylinder turbo charged 145 hp @ 2300 rpm

7.	Torque 420 lb. ft. @ 1600 rpm

8.	Automatic Transmission (M74) Allison 1000 Series with overdrive

9.	Brakes hydraulic power 4-wheel disc

10.	Parking brake cable to rear wheels up to 11,000 lb. GVWR transmission mounted drum on 12,000 and 14,100 lb. GVWR hand operated lever control

11.	Steering integral power Saginaw 708 ratio 14:1

12.	Frame maximum resisting bending moment 177,600 lb.; inner reinforcements

13.	Front Axle 5,000 lb. capacity I-Beam axle (11,000-12,000 and 14,100 lb.)

14.	Rear Axle 11,000 lb. ratio 4:63 GVWR 11,000 to 14,100 lb

15.	Front suspension stabilizer bar (2.5 in. square) and Shocks (32mm piston diameter) 4400 lb. and 5,000 lb. parabolic leaf suspension

16.	Rear suspension shocks 35 mm piston diameter parabolic leaf springs

17.	Tach and trip odometer, dual note electric horn, tilt steering column with anti-theft locking feature and custom steering wheel, pulse-wipe, cruise control, wiper/washer control, headlight dimmer, brake/transmission shift interlock, 40 gallon fuel tank, 130 AMP alternator, electronic convenience center

18.  	3-point seatbelt

19. 	Heater in load compartment

20.	Rear door vents and/or glass

Bid Specs

Walk-In Van Sheriff’s Department

Page Two

21.	Grab handles

22.	Interior lights (in addition to standard)

23. 	Exterior mirrors (other than standard)

24.	Grated rear bumper

25.	Roof vents and skylights

26.	Specify warranty

REQUIREMENTS:

Real Estate and Personal Property Taxes:  ANo bid will be accepted or opened on any County contract if the vendor is listed on the last published list of delinquent real or personal property taxes in Kanawha County; however, the Commission will accept bids by vendors who provide satisfactory proof of payment of current taxes or a certification from the Sheriff that no taxes are due prior to submission of said bid.@

Required Forms:     Vendor shall complete and submit, or have on file with the County, a Vendor Registration and Disclosure Statement Form.  A registration form is being supplied for your convenience.