KANAWHA COUNTY COMMISSION

P. O. BOX 3627

407 VIRGINIA STREET, EAST

CHARLESTON, WEST VIRGINIA 25336

(304) 357-0117

Request for Quotations

Re:	 Rescue Tools for the Handley Volunteer Fire Department

Date:	February 28, 2005

Fiscal Year:	2004-05

Bid Opening:	Bids must be received on or before Monday, March 14, 2005, at 2:00 p.m. in the Kanawha County Commission Purchasing Office, 407 Virginia Street, East, Third Floor, Room 229, Charleston, WV 25301 (P.O. Box 3627, Charleston, WV 25336)

INSTRUCTIONS TO BIDDERS:

*PLEASE USE THIS FORM AS THE COVER SHEET FOR YOUR BID

1.	Bids must be received in a sealed envelope with the date and time of the bid opening on the outside of the envelope.  Faxed bids will not be accepted.

2.	Bid must be F.O.B. Delivery Point, unless otherwise indicated in proposal.

3.	All bids should be signed and in ink, showing all facts and the total amount of the bid.

4.	The Handley Volunteer Fire Department reserves the right to accept or reject in part or in whole any bid submitted, whichever is in the best interest of Handley Volunteer Fire Department.

| Item No. | Description | Model # | Amount |
| --- | --- | --- | --- |
| 1 | One (1) Simo Power Unit per Attached Specifications |  | $ |
| 2 | One (1) 32" Spreader per Attached Specifications |  | $ |
| 3 | One (1) Heavy Duty Cutter per Attached Specifications |  | $ |
| 4 | Two (2) Diamond Tips for Spreader per Attached Specifications |  | $ |
| 5 | One (1) Telescoping 41" Ram per Attached Specifications |  | $ |
| 6 | One (1) 30' Hose Extension per Attached Specifications |  | $ |

Vendor Name:  _____________________________     	Signature: __________________________

Address:           _____________________________     	Date:         __________________________

_____________________________  	Telephone: _________________________

***ATTENTION VENDORS***

Have you registered for the Kanawha County Commission Automatic Vendor E-Mail Notification System for bid specifications?  Register by visiting our website @                     www.kanawhacounty.com/purchasing/alerts .

SPECIFICATIONS

COUNTY COMMISSION OF KANAWHA COUNTY

CHARLESTON, WEST VIRGINIA

ITEM:			Hydraulic Rescue Tools for the Handley Volunteer Fire Department

LOCATION:		Handley Volunteer Fire Department

Route 61, Second Avenue, #23

Handley, WV 25102

CONTACT:		Jerie Whitehead			James “Bear” Black

Purchasing Director			Handley Volunteer Fire Department

Kanawha County Commission	P.O. Box 231

407 Virginia St., East		Handley, WV 25102

P.O. Box 3627			Telephone (304) 442-5712

Charleston, WV 25336

Telephone (304) 357-0115

BID OPENING:	Bids must be received in a sealed envelope, with the date and time of the bid opening on the outside of the envelope, on or before Monday, March 14, 2005, at 2:00 p.m., in the Kanawha County Commission Purchasing Office, 407 Virginia Street, East, Third Floor, Room 229, Charleston, West Virginia 25301 (P.O. Box 3627, Charleston, WV 25336).  Faxed bids will not be accepted.

SPECIFICATIONS:	The following specifications are intended to describe hydraulic rescue tools for the Handley Volunteer Fire Department and the details contained in these specifications are not designed to exclude any vendor from bidding, but are offered as a means of describing the needs of the Handley Volunteer Fire Department.  Where brand names may be used, the words “or equal” are assumed to follow.  All specifications are minimum requirements.

(1) SIMO Power Unit

1.	The power unit will consist of a hydraulic pump in a reservoir driven by an engine.  The reservoir capacity must be a minimum of 2.1 gal. (8 liters)

Comply ______N          _______Y

2.	The engine is to be a 6.5 HP Honda engine with electronic ignition.  Muffler is to have a spark arrestor.

Comply ______N        _______Y

3.	The hydraulic pump is to be a two-stage radial piston pump.  The pump is to be capable of producing 10,000 psi (690 bar).  The system will have a maximum operating pressure of 5,000 psi (345 bar) to provide long pump life.  The low pressure stage shall shift at 2000 psi (138 bar) for faster operation.

Comply ______N        _______Y

4.	The unit is to provide power to operate two tools simultaneously.  There shall be two separate dump valves.  Each tool is to operate independently and obtain full pressure and flow at the same time.

Comply _______N      ______Y

5.	The unit is to have four short hoses permanently attached with quick disconnects for adding additional lengths of hose.

Comply ______N       ______Y

Bid Specifications

Handley VFD Rescue Tools

Page Two

6.	The engine and reservoir are to be protected by a tubular steel roll cage which must surround the engine and reservoir.

Comply ______N       ______Y

7.	The power unit is to have anti-vibration isolators to limit unwanted movement.

Comply ______N       ______Y

8.	The power unit dimensions are to be 18.9 in (480 mm) by 17.22 in (437 mm) by 19.69 in. (500 mm).

Comply ______N       ______Y

9.	The power unit shall not weigh more than 87 lbs. (39.5 kg)

Comply ______N       ______Y

(1) Heavy Duty Cutter

1.	The cutter opening will be no less than 6.19 in (159 mm).  The cutter  will be of a curved design and made so as everything is pulled to the rear of the blades and cut.  Front of blades cross before rear, pulling everything to the strongest cut position.

Comply ______N       ______Y

2.	The cutter is to produce a minimum of 38,000 lb (172 kn) of cutting force at the center of the blade and no less than 72,000 lb (319 kn) at the rear.  (Handley VFD reserves the right to test cut pressure with a dynamometer strain gauge or with raw steel).

Comply ______N       ______Y

3.	The blades of the tool should be attached to the piston rod via removable links, for ease of repair, efficient power transmission and smooth operation.

Comply ______N       ______Y

4.	The body of the tool is to be of aluminum forging for its light weight, strength and long life as per Section 2.

Comply ______N       ______Y

5.	The tool is to have two handles.  One located at the center of the tool and the other located below the control mechanism.

Comply ______N       ______Y

6.	The tool is to have a dual pilot check valve to prevent accidental movement of the arms in the event of power loss.

Comply ______N       ______Y

Bid Specifications

Handley VFD Rescue Tools

Page Three

7.	The control mechanism is to be actuated by thumb pressure.  The mechanism is to be protected on both sides to prevent jamming during operation.  The mechanism must be of deadman design, whereby the unit stops functioning when thumb pressure is released.

Comply ______N       ______Y

8.	The tool is to have two (2) 20 in (508 mm) hoses attached with quick pin lock disconnects on the ends for the attachment of additional hose lengths.

Comply ______N       ______Y

9.	The tool shall be as compact as possible.  It must not be longer than 28.8 in (733 mm) higher than 7.3 in (185 mm) or wider than 11 in (280 mm).

Comply ______N       ______Y

10.	The tool is to weigh 39.4 lb (17.87 kg) or less.

Comply ______N       ______Y

Include a copy of NFPA compliance certification from UL with bid.

(2) Extension Hoses

1.	Hose must be 30' length extensions.  Hose must be non-conductive, thermoplastic, twin lined Kevlar hose.

Comply ______N       ______Y

2.	Hose must be light weight, kink resistant, permanently bonded and provide superior flexibility.

Comply ______N       ______Y

3.	Hose must be completely synthetic and Kevlar reinforced.  Must meet or exceed SAE 100R8.  Must be proof tested to 20,000 psi and a 4 to 1 operational safety factor.

Comply ______N       ______Y

4.	Hose must have two (2) sets of couplings

Comply ______N       ______Y

(1) Diamond Tips for Spreaders

1.	Tips are to be black anodized in color; they are to be made from investment cast heat-treated steel.

Comply ______N       ______Y

2.	Tips of the tool must be removable for ease of repair and for the accommodation of optional tips.  They are to be held in place by a field replaceable detent ball pin or screw.

Comply ______N       ______Y

3.	The tips should have a pointed cross section design on the entire outer surface area and on the upper half of the inside surface area.

Comply ______N       ______Y

Bid Specifications

Handley VFD Rescue Tools

Page Four

4.	The tip should have an additional hole for quick attachment of chain and shackles or other optional components using a shackle pin as the attachment device.

Comply ______N       ______Y

(1) 32" Heavy Duty Spreader

1.	Spreader shall have no less than 32" of spreading distance.

Comply ______N       ______Y

2.	Spread force at tips to be no less than 16,000 lbs. and pulling force of no less than 14,400 lbs. (Handley VFD reserves the right to test spreader force with a dynamometer strain gauge).

Comply ______N       ______Y

3.	Tool weight to be 52 lbs. with fluid and connectors.

Comply ______N       ______Y

4.	Tool to be 27.3" long, 14" wide and 8.3" deep

Comply ______N       ______Y

(1) Telescoping Ram

1.	Ram shall have dual pilot check valve for safe load handling.

Comply ______N       ______Y

2.	Claw style foot design to reduce slippage and provide safe operation.  Knurled actuator handle for a tight non-slip grip and easy transportability.

Comply ______N       ______Y

3.	Ram length retracted to be 18.7".

Comply ______N       ______Y

4.	Ram push force to be no less than 29,830 lbs. with first piston, and 14,830 lbs. with second piston.  (Handley VFD reserves the right to test tool pressures with a dynamometer strain gauge).

Comply ______N       ______Y

5.	Ram to have a total extended length of 41.3".

Comply ______N       ______Y

6.	Ram to weigh no more than 40 lbs. with fluid connectors.

Comply ______N       ______Y

Bid Specifications

Handley VFD Rescue Tools

Page Five

Section 1 	Specifications for Hydraulic Rescue Tools

1.1	These specifications for the previosuly listed rescue tools require new, professionally produced hydraulic rescue tools

1.2	All parts and accessories which are necessary for proper operation of the equipment should be included.  All equipment listed shall be of current models and equipment in production and use.

1.3	All system components shall be interchangeable with other system components of the same manufacture, and capable of interconnecting with other service tools now in use or to be used in the future  by the Handley Volunteer Fire Department or mutual aid departments.

1.4	Due to the use of the specified rescue tools in emergency and hazardous situations, the equipment listed must be held in strict compliance with the specifications.   Successful bidder will be responsible for delivery of equipment that meets or exceeds these specifications.

1.5	All equipment must be NFPA 1936 compliant and bidders are required to submit with their bid copies of the UL certification of compliance to include issue dates and certificate numbers.

1.6	All equipment listed must be certified by a minimum of five (5) additional globally recognized testing certifications (U.S. Air Force Compliance

Testing PD-89-VRG08, U.S. Navy Certification Training, French Rescue Tool Standard NF S 61-571, MIL 1-45208A, ISO 9001, ISO 14001 or SAE J517C 100R-8).

1.7	Bidders are required to provide two (2) sets of detailed specifications for the equipment  being proposed in their bid documents.

1.8	A minimum of three (3) references from customers who currently use the proposed equipment are required to be submitted with the bid documents.  References are to include a contact person, the name of the company/organization, the address and telephone number.

1.9	Training is to be provided by an authorized factory training representative and is to be included in the quotation.

Section 2	Construction Materials and Methods of Production and Warranties

2.1	The aluminum used in the construction of the cutter, ram bodies, cylinders, and frames of the spreaders shall be of 7075 T6 aluminum with an 83,000 lb. tensile strength.   The metal is to first be forged, then tempered and machined.

Comply ______N	______Y

2.2	All cutters, ram bodies, cylinders and frames of the spreaders are to have helicoils in the threads of all bolt circles for added thread strength.

Comply ______N	______Y

2.3	All tools are to have forged alloy steel pistons and piston rods to resist bending and deforming.

Comply ______N	______Y

2.4	Cutter blades are to be made of shock-resistant tool (S series) forged, tempered and then machined steel.  Rockwell hardness of steel shall be of Rockwell 53 C Scale

Comply ______N  	______Y

Bid Specifications

Handley VFD Rescue Tools

Page Six

2.5	Tools are to operate with MCS2361 fluid.  Fluid is to be fire resistant, non-corrosive, electrically non-conductive, non-toxic, and be a rust preventative lubricant that maintains its viscosity over a wide range of temperatures.  The Handley Volunteer Fire Department reserves the right to request an MSDS sheet.

Comply ______N  	______Y

2.6	All equipment must have a minimum two-year guarantee on all parts.

Comply ______N    ______Y

Equipment Bids

(1)	Simo Power Unit

Model #	  _________________

Amount	$_________________

(1) 	Heavy Duty Cutter

Model	#	  _________________

Amount	$_________________

(2)	30‘ Hose Extensions

Model	#	  _________________

Amount	$_________________

(1)	Diamond Tips for Spreaders

Model	# 	  _________________

Amount	$_________________

(1) 	32" Heavy Duty Spreader

Model	#	  _________________

Amount 	$_________________

(1)	Telescoping 41" Ram

Model	#	  _________________

Amount	$_________________

Dealer is required to have loaner tools available for use by the Handley VFD while equipment may be unavailable during service.  Dealer must also have certified technicians available.

State the hours technicians are available and service rates (if applicable)

Hours: ____________________________________________________

Rates: ____________________________________________________

REQUIREMENTS:

Delinquent Real Estate and/or Personal Property Taxes:

“No bid will be accepted or opened on any County contract if the vendor is listed on the last published list of delinquent real or personal property taxes in Kanawha County; however,  the Commission will accept bids by vendors who provide satisfactory proof of payment of current taxes or a certification from the Sheriff that no taxes are due prior to submission of said bid.”

Required Forms:     Vendor shall complete and submit, or have on file with the County, a Vendor Registration and Disclosure Statement Form.

Rejection of Bids:  The Kanawha County Commission reserves the right to reject any and/or all bids and to waive any informalities in bidding.  All bid documents become public record upon opening.