KANAWHA COUNTY COMMISSION

P. O. BOX 3627

407 VIRGINIA STREET, EAST

CHARLESTON, WEST VIRGINIA 25336

(304) 357-0115

Request for Quotations

Re:		Provide and Install a 7.5 Ton Split System Heat Pump in the Kanawha County Clerk’s Record             Room

Date:		January 25, 2005

Fiscal Year:		2004-05

Bid Opening:	Bids must be received on or before Monday, February 14, 2005, at 2:00 p.m. in the Kanawha County Commission Purchasing Office, 407 Virginia Street, East, Third Floor, Room 229, Charleston, WV 25301 (P.O. Box 3627, Charleston, WV 25336)

INSTRUCTIONS TO VENDORS:

*PLEASE USE THIS FORM AS THE COVER SHEET FOR YOUR Bid

1.	Bids must be received in a sealed envelope with the date and time bids are due on the outside of the envelope.  Faxed bids will not be accepted.

2.	Bid must be F.O.B. Delivery Point, unless otherwise indicated in bid.

3.	All bids should be signed and in ink, showing all facts and the total amount of the bid.

4.	The Kanawha County Commission reserves the right to accept or reject in part or in whole any bid submitted, whichever is in the best interest of the Kanawha County Commission.

| Item No. | Description | Bid Amount |
| --- | --- | --- |

| 1 | Provide and Install a 7.5 Ton Split System Heat Pump in the Kanawha County Clerk’s Record Room per the Attached Specifications | $ |
| --- | --- | --- |

Written Bid Amount: _________________________________________________________________ _____________________________________________________Dollars

Vendor Name:  ___________________________________  Signature:  __________________________________

Address:          ___________________________________  Date:          __________________________________

___________________________________  Telephone:  _________________________________

***ATTENTION VENDORS***

Have you registered for the Kanawha County Commission Automatic Vendor E-Mail Notification System for bid specifications?                                      	Register by visiting our website @ www.kanawhacounty.com/purchasing/alerts .

SPECIFICATIONS

COUNTY COMMISSION OF KANAWHA COUNTY

CHARLESTON, WEST VIRGINIA

ITEM:	Provide and Install a 7.5 Ton Split System Heat Pump in the Kanawha County Clerk’s Record Room

LOCATION:	Kanawha County Clerk’s Record Room

409 Virginia Street, East, First Floor

Charleston, WV 25301

CONTACT:		Jerie Whitehead			Darrell Savage

Purchasing Director			Maintenance Supervisor

Kanawha County Commission	Kanawha County Commission

407 Virginia St., East		P.O. Box 3627

P.O. Box 3627			Charleston, WV 25336

Charleston, WV 25336		Telephone (304) 357-0106

Telephone (304) 357-0115

BID OPENING:	Bids must be received in a sealed envelope, with the date and time of the bid opening on the outside of the envelope, on or before Monday, February 14, 2005, at 2:00 p.m., in the Kanawha County Commission Purchasing Office, 407 Virginia Street, East, Third Floor, Room 229, Charleston, West Virginia 25301 (P.O. Box 3627, Charleston, WV 25336).  Faxed bids will not be accepted.

SPECIFICATIONS:	The following specifications are intended to describe providing and installing a 7.5 ton split system heat pump in the Kanawha County Clerk’s Record Room and the details contained in these specifications are not designed to exclude any vendor from bidding, but are offered as a means of describing the needs of the Kanawha County Commission.  Where brand names may be used, the words “or equal” are assumed to follow.  All specifications are minimum requirements.

1.	Provide and install a 7.5 ton split system heat pump for the Kanawha County Clerk’s Record Room.

2.	Remove and properly dispose of the existing air handler located in the Kanawha County Clerk’s Record Room ceiling.  Work is to be performed on a weekend (Saturday and Sunday) and may be completed on Monday, but  contractor will be responsible for securing the work area so the public will have access to records.  Scheduling is required in advance.

- Install a new Trane Model TWA090A300F (or equivalent) air handler, to include an auxiliary drain pan, in the Kanawha County Clerk’s Record Room ceiling.  The unit is to be installed with sufficient space for routine maintenance to be performed on the unit.  The existing unit is not installed to allow for sufficient space for routine maintenance to be performed.  Work is to be performed on a weekend (Saturday and Sunday) only.  Scheduling is required in advance

- Remove and properly dispose of the existing outdoor condensing unit located on the Court Street side of the Kanawha County Courthouse.  Provide required crane service for setting the outdoor unit.

- Install a new Trane Model TWA090A300F (or equivalent) outdoor condensing unit on the Court Street side of the Kanawha County Courthouse.  The unit is to include a variable condenser fan speed low ambient control.  Simple fan cycling

Bid Specifications

Provide and Install 7.5 Ton Split System Heat Pump

Page Two

control is not acceptable.  The unit is to include a 3-phase line monitor that         protects against phase loss, phase reversal and phase unbalance.

6.	The existing refrigeration piping between the indoor and outdoor equipment can be reused.

7.	Install a new 2-stage thermostat.

8.	Install a new 240 volt, 3 phase, 60 amp electrical disconnect for the outdoor unit.

9.	The electrical disconnect serving the indoor unit can be reused.

10.	Modify the existing supply and return air duct system to allow unit to be installed with sufficient space for routine maintenance to be performed on the unit.  Unit is to be positioned to allow the blower and heater kit to be easily accessed and removed if necessary.  All ducts will be of metal construction.  Duct board is not acceptable.

11.	An HVAC Permit from the City of Charleston is required to be obtained by the Contractor.  There is no fee for the permit.

12.	State lead time to obtain the equipment.

13.	Provide manufacturer and model for equipment bid:

Air handler:

Manufacturer ___________	Model ______________________

Outdoor condensing unit:

Manufacturer ___________	Model ______________________

14.	State of West Virginia Prevailing Wage Rates, as established by the most recent publication of those rates, are required to be paid by the Contractor and all Sub-Contractors.   A copy of the Certified Payroll is to be submitted with the invoice for payment.

15.	Provide a copy of a current West Virginia Contractor’s license with bid documents.

16.	Successful bidder is required to submit the following prior to commencement of work:

A.	Proof of a minimum one million dollar per occurrence commercial, general liability insurance policy with the Kanawha County Commission named as additional insured.

- References of previous commercial work from three Charleston area businesses are required.

- Worker’s Compensation and Unemployment Premiums: Successful bidder must provide satisfactory certification from Workers’ Compensation and the Commissioner of Employment Security as to current status of vendor’s Workers’ Compensation and Unemployment Security premiums.

Bid Specifications

Provide and Install 7.5 Ton Split System Heat Pump

Page Three

- Vendor shall complete and submit, or have on file with the County, a Vendor Registration and Disclosure Statement Form and a No Debt Affidavit.

17.	Standards of Quality and Codes: All work, labor, materials and equipment  specified, constructed and installed are to be of first class quality.  To help  insure this occurs, all work and equipment designed and specified shall conform to the latest applicable codes and standards:

A.  Building Officials & Code Administrators (BOCA)

B.  American Society of Testing Materials (ASTM)

C.  American National Standards Institute (ANSI)

D.  National Fire Protection Association (NFPA)

E.  National Electric Code (NEC)

All work must also have the approval of all WV governmental authorities and agencies having jurisdiction over the project:

1.  WV State Fire Marshall

2.  WV Department of Health

3.  WV Department of Natural Resources

For the Owner’s Record, submit copies of permits, licenses, certifications, inspection reports, releases, notices, receipt for fee payments, correspondence and records established in conjunction with compliance with standards and regulations bearing upon performance of work.

18.		Safety & Job Site Requirements:  All Federal Occupational Safety & Health Administration Regulations must be followed by all contractor personnel while performing work for the Kanawha County Commission.  All work as described or required shall be executed in neat, skillful, workmanlike manner in accordance with the best recognized trade practices.  Only competent workmen who satisfactorily perform their duties shall be employed on this project.  Contractor shall discharge and shall not re-employ on this project, any person who is disorderly, dangerous, insubordinate, incompetent or otherwise objectionable or who uses alcohol or illicit drugs on the job site.

19.	Real Estate and Personal Property Taxes: “No bid will be accepted or opened on any County contract if the vendor is listed on the last published list of delinquent real or personal property taxes in Kanawha County; however,  the Commission will accept bids by vendors who provide satisfactory proof of payment of current taxes or a certification from the Sheriff that “no taxes are due prior to submission of said bid.”

20.	Rejection of Bids:  The Kanawha County Commission reserves the right to accept or reject, in part or in whole, any and/or all bids submitted, whichever is in the best interest of the Kanawha County Commission, and to waive any informalities in bidding.

Attachment A

Provide and Install 7.5 Ton Split System Heat Pump in the Kanawha County Clerk’s Record Room

Bids Due Monday, February 14, 2005, 2:00 p.m.

I.	Documents required to be submitted with Bid:

1.	Copy of a current West Virginia Contractor’s License

II.	Provide the following information with your bid:

1.	Lead time to obtain the equipment: ____________

2.	Provide manufacturer and model for equipment bid:

Air handler:

Manufacturer ___________  Model ______________________

Outdoor condensing unit:

Manufacturer ___________  Model ______________________

III.	Documents to be submitted by successful bidder:

1.	    Proof of a minimum one million dollar per occurrence commercial, general       liability insurance policy with the Kanawha County Commission named as        additional insured.

2.	    References of previous commercial work from three Charleston area                 businesses.

3.	       Provide satisfactory certification from Workers’ Compensation and the Commissioner of Employment Security as to current status of vendor’s Workers’ Compensation and Unemployment Security premiums.

4.		Vendor shall complete and submit, or have on file with the County, a “Vendor Registration and Disclosure Statement” and a “No Debt Affidavit”.