KANAWHA COUNTY COMMISSION

P. O. BOX 3627

409 VIRGINIA STREET, EAST

CHARLESTON, WEST VIRGINIA 25336

(304) 357-0117

Request for Bids

Re:			Audio System for Commission Courtroom

Date:			September 22, 2005

Fiscal Year:		2005-06

Bid Opening:		Bids must be received on or before Friday, October 21, 2005, at 11:00 a.m. in the Kanawha County Commission Purchasing Office, 407 Virginia Street, East, Third Floor, Room 229, Charleston, WV 25301 (P.O. Box 3627, Charleston, WV 25336)

INSTRUCTIONS TO BIDDERS:

*THIS FORM MUST BE THE COVER SHEET FOR YOUR BID

1.	Bids must be received in a sealed envelope with the date and time of the bid opening on the outside of the envelope.  Faxed bids will not be accepted.

2.	Unsigned bids will not be accepted.

3.	Bid must be F.O.B. Delivery Point, unless otherwise indicated in proposal.

4.	All bids should be in ink, showing all facts and the total amount of the bid.

5.	The County reserves the right to accept or reject in part or in whole any bid submitted, whichever 	is in the best interest of the County.

| Item No. | Quantity | Description | Amount |
| --- | --- | --- | --- |

| 1 2 3 4 5 6 7 8 9 10 11 12 | 1 1 1 1 1 1 1 1 1 1 1 8 | ALL ITEMS MUST COMPLY WITH APPENDIX A Solid-State Digital Audio/Video 16 Channel Audio with automatic mixer/switcher, Computer Remote Controllable Line Protector w/4 Duplex Outlets Press/Media feed for press and Commission IT Audio Distribution Amplifier for Press/Media feeds Press Feed Plate with 4 A/V outputs Dual channel 150 watts, 70/100 volt power amplifier DBX Dual Channel Advanced feedback Suppression Processor Network Audio Adapter, audio routing over LAN or Internet Wiring, transformers and/or amps for carrying audio to Commission IT server room Solid-State Digital Audio Recorder w/edit software Wood veneer locking rack for all equipment. Replace existing mics with high performance mics |  |
| --- | --- | --- | --- |

Vendor Name:  _____________________________     		Signature:______________________

Address:           _____________________________     		Date:  	_____________________

_____________________________  		Total Bid:  $______

Telephone:       _____________________________

SPECIFICATIONS

COUNTY COMMISSION OF KANAWHA COUNTY

CHARLESTON, WEST VIRGINIA

ITEM:			Audio System for Commission Courtroom

LOCATION:		Kanawha County Commission

409 Virginia Street East

Charleston, WV 25301

CONTACT:		Jerie Whitehead			Dennis Wyer

Purchasing Director			System Administrator

Kanawha County Commission	Kanawha County Commission

407 Virginia St., East	          Goshorn Street

P.O. Box 3627			Charleston, WV 25301

Charleston, WV 25336		Telephone (304) 357-9810

Telephone (304) 357-0115

BID OPENING:	Bids must be received in a sealed envelope, with the date and time of the bid opening on the outside of the envelope, on or before Friday, October 21, 2005, at 11:00 a.m., in the Kanawha County Commission Purchasing Office, 407 Virginia Street, East, Third Floor, Room 229, Charleston, West Virginia 25301 (P.O. Box 3627, Charleston, WV 25336).  Faxed bids will not be accepted.

SPECIFICATIONS:

Quote to provide digital audio/video recording system for Kanawha County Commissioners Hearing Room to capture to MPEG-4 files for storage to disk and archive to CD-R and for live output to the LAN/Internet.

Eight (8) high performance microphones to replace existing units; Commissioner’s positions to have push-to-mute feature.  Room sound to be upgraded with new amplifier, feedback suppressor and additional ceiling speakers.  Audio/Video output for press/media connection provided in the courtroom.  System will connect to existing hearing impaired equipment.

TERMS & CONDITIONS:

Real Estate and Personal Property Taxes:  No bid will be accepted or opened on any County contract if the vendor is listed on the last published list of delinquent real or personal property taxes in Kanawha County; however, the Commission will accept bids by vendors who provide satisfactory proof of payment of current taxes or a certification from the Sheriff that no taxes are due prior to submission of said bid.

Required Forms:  Vendor shall complete and submit, or have on file with the County, a Vendor Registration and Disclosure Statement Form.  A registration form is being supplied for your convenience.

Rejection of Bids:  The Kanawha County Commission reserves the right to reject any and/or all bids and to waive any informalities in bidding.

APPENDIX - A

# Part 1 - GENERAL:

## 1.1 SUMMARY

- This document describes a Microphone Control and Sound Reinforcement system preformance specification.  It includes requirements for Microphone Control and Sound Reinforcement system components including, but not limited to, the following:

  - Microphones and accessories

  - Power Amplifiers

  - Microphone Receptacles

  - Wiring

  - Loudspeakers

  - Audio Signal Mixers and Control Components

  - Recording Equipment

  - All required components for proper impedance matching, signal balancing, and data conversion software required for the proper operation of the Microphone Control and Sound Reinforcement systems

  - Miscellaneous material as required

## 1.2 SYSTEM DESCRIPTION

- General: The Microphone Control and Sound Reinforcement system shall be a complete system for amplifying sound signals from microphone outlets and from other “line level” signals such as a cassette player or CD player and distributing them to the loudspeakers.

- The Microphone Control and Sound Reinforcement system shall be furnished with all required equipment, multiple inputs, interconnect cabling, interconnective termination strips, as required, final equipment knob/control/microphone location labeling, and assemblies as required for a fully functional system allowing each feature to operate properly.

- System contractor shall be responsible to furnish and install all cable, conduit, speakers, microphones, system head end and all other “field” equipment as indicated on the project plans and specified herein, as directed and supervised by the equipment supplier.  The System Contractor shall contract the professional services, and equipment of a sound system supplier that can provide the final testing procedures, system interfaces, in-service training for the owner’s representative, and signal flow diagrams as required in this specification section.  The System Contractor shall contract the services of a qualified electrical contractor (if necessary) to provide all necessary power requirements.  Final terminations of the field equipment to be performed by the System Contractor as directed by the equipment supplier’s detailed drawings.  System Contractor shall be responsible to procure these services as required, and oversee and coordinate the smooth and efficient integration of these systems with all parties concerned.  System Contractor shall provide a copy of their WV contractor’s license with proposal.

- Functional Performance: Components and system features and functions shall include, but are not limited to, the following:

- Multiple Sources: The system shall have switch selectability of sources for sound amplification between various microphones and inputs designated and arranged for cassette deck, CD player and auxiliary equipment.

- High-Quality Sound Reproduction: Freedom from noises such as pops, clicks, hiss and hum at all loudspeakers at all times during operation of the system, including standby mode with inputs off.  Freedom from distortion, feedback and non-uniform coverage of amplified sound shall be required.

- Digital signal processing:  The system shall include digital signal processing to enhance the overall quality of the sound being reproduced.

- Assistive Listening:  The system shall include an assistive listening system for the hearing impaired.  Detailed requirements are listed below.

- Press Box:  A press box assembly shall be provided to allow public recording by the press.

- Telephone connection:  The system shall provide a means of connecting a live telephone signal to the system for the purpose of teleconferencing.

- Muting:  Mute buttons (kill switch) shall be provided for the microphones on the counsel tables.

## 1.3 SUBMITTALS

    - Product data specification sheets for each product specified, to be provided with proposal.

    - Successful System contractor shall submit within two weeks after acceptance of contract: shop drawings detailing Microphone Control and Sound Reinforcement system for Owner approval; including, but not limited to the following:

- System cable riser diagram.

- System Interconnect signal and control diagrams showing all logic and data control features.

- Wattage and voltage tap information for speakers.

- Cable Termination information for each component in the system.

- Wiring Diagrams detailing wiring for power, signal, and control differentiating clearly between manufacturer-installed wiring and field-installed wiring.  Identify terminal numbers and wiring color codes to facilitate installation, operation, and maintenance.

- Maintenance data for materials and products, for inclusion in Operating and Maintenance Manual specified.

## 1.4 QUALITY ASSURANCE

  - Installer Qualifications: Engage an experienced Installer with supervision from a factory-authorized sales and service representative for the project location to perform the work of this section. The audio equipment supplier shall be a factory authorized distributor for the products supplied.  The audio company shall have been in business a minimum five years.  The audio equipment supplier shall be able to provide a list of similar systems that have been installed over the past five years.  The Owner may visit to verify upon request.  Upon request, submit evidence of such qualifications to the Owner.

  - Electrical Component Standard: Provide work complying with applicable requirements of NFPA 70 “National Electrical Code.”

  - EIA Compliance: Comply with the following Electronics Industries Association Standards:

    - Sound Systems, EIA-160.

    - Loudspeaker, Dynamic Magnetic Structures, and Impedance, EIA-229-A.

    - Racks, Panels, and Associated Equipment, EIA-310-A.

    - Amplifiers for Sound Equipment, SE-101-A.

    - Speakers for Sound Equipment, SE-103.

    - Microphones for Sound Equipment, SE-105.

## 1.5 DELIVERY, STORAGE, AND HANDLING

- Deliver products in factory containers.  Store in clean, dry space in original containers.  Protect products from fumes and construction traffic.  Handle carefully to avoid damage.

# Part 2 - PRODUCTS:

## 2.1 MANUFACTURERS

- Equipment brands that are named in this specification set the desired minimum performance quality level for all associated features and functions applicable to the model numbers specified.  This specification further lists the minimum performance characteristics, features, and functions of all other equipment for this project.  Any substitutions to the listed equipment must be submitted along with the manufacturer’s cut sheets prior to bid.  Any sbustitutions must meet or excceed the specifications for the equipment listed herein.

- The System contractor shall be held responsible for the equipment meeting these specifications. The equipment shall be formally approved at the final system acceptance after the equipment has been installed, terminated, and ready for inspection by the Owner.  It shall be the system contractor’s responsibility to remove, replace, and reinstall all equipment, at no cost, found to be in non-compliance with these specifications at the time of final acceptance and testing by Owner.  Submittal review and acceptance by the Owner of this equipment shall be preliminary.  The endorsement or approval of the equipment submittal by the Owner shall not relieve the system contractor of this responsibility to remove, replace, and re-install all equipment at no cost to meet this specification.

- Equipment Manufacturers Standard of Acceptance:

      - Atlas-Soundolier

      - Audio-Technica

      - Communications Company, Inc.

      - Electro-Voice

      - Lectrosonics

      - Lowell

      - Middle Atlantic

      - Shure

      - TEAC

      - Telex

      - Yamaha

## 2.2 SYSTEM REQUIREMENTS

- Provide complete and fully functional Microphone Control and Sound Reinforcement systems using materials and equipment of types, sizes, ratings, and performances as indicated.  Use materials and equipment that comply with referenced standards and manufacturers’ standard design and construction in accordance with published product information.  Coordinate the features of materials and equipment so they form an integrated system with components and interconnections matched for optimum performance of specified functions.  The functional Microphone Control and Sound Reinforcement consists of: mixer/amplifier, system control components, interface circuits/equipment for the microphone control systems, multiple audio inputs, speakers, fixed microphones, multiple microphone and line level inputs, and provision for connecting recording equipment.

## 2.3 EQUIPMENT AND MATERIALS

- Provide equipment using all solid-state components fully rated for continuous duty at the ratings indicated or specified.  Select equipment for normal operation on input power supplied at 105-130 V, 60 Hz.

  - Automatic microphone mixer The base system shall be comprised of a multi-input automatic microphone mixer with either an integrated or an external crosspoint matrix.  The automatic mixer shall be capable of “mix-minus” sound reinforcement, shall be remote controllable either via a LAN computer (RS-232) interface or an industry standard control system such as CRESTRON, shall have switchable phantom power on each channel, shall have a minimum of eight (8) microphone inputs and a minimum of two (2) line level inputs (for external audio sources), and shall be expandable or “stackable” to allow for future expansion of the base system. Lectrosonics DM1612

  - Crosspoint Matrix The base system shall also contain a crosspoint matrix capable of being remotely controlled as noted above.  The matrix shall provide a minimum of four (4) line level outputs (+4dB) with adjustable gain on each channel and shall be expandable. Lectrosonics MM8

  - Telephone Hybrid A digital telephone hybrid interface that incorporates echo canceling, selective muting, auto-answer/disconnect, automatic gain control on both transmit and receive, DTMF dialing, a wired remote, and serial control in one unit shall also be provided with the base system.  The telephone hybrid should provide a means of interfacing with the automixer in such a way as to seamlessly integrate into the “mix-minus” operation of the automixer. Lectrosonics TH3A

  - Digital Signal Processor The base system shall include a programmable equalizer/digital signal processor (DSP) with the following features:

        - 2x2 matrix (nom.)

        - compression and limiting on each channel

        - adjustable delay on each channel

        - programmable inputs and outputs

        - remote control as noted above

        - automatic feedback control

        - 12 filters per channel (min.): HP, LP, shelving, parametric EQ

Shure DFR22

  - Power Amplifier A power amplifier shall be required for accurate reproduction of audio signals.  The amplifier shall provide a minimum of 6 input channels and 6 output channels and be capable of simultaneous constant voltage and low impedance operation.  The output power of the amplifier shall be 100W (nominally) for constant voltage operation and 100W (nominally) for low impedance operation.  The power amplifier shall also meet the following minimum requirements:

Input Impedance:		10 to 30 k ohms (balanced)

15 to 37 k ohms (unbalanced)

Frequency Response:		20 to 20 kHz (minimum)

Intermodulated Distortion:	<0.2% (SMPTE 4:1)

Total Harmonic Distortion:	<0.2%

S/N Ratio:			100dB or greater

Sensitivity:			0dB min. (rated power, 8ohms)

Voltage Gain:			30dB (min.)

Connectors:

Input:			Euroblock, XLR, or ¼” balanced per ch.

Output:			5 way binding post

The amplifier shall also incorporate protection circuitry to protect loudspeakers from transients and spikes in the audio signal and shall have a switchable HP filter on each input. Yamaha 6150

  - Assistive Listening Devices The base system shall incorporate assistive listening devices that may be either RF type or Infra-Red type dependent upon site requirements and/or restrictions.  The assistive listening system shall provide a minimum of 1 in ear (or headphone) type headset and 1 neck-loop (T-coil) type.  The base station (transmitter) shall provide both ¼” and XLR type inputs for interfacing with the automixer. Telex Soundmate SM2

In the case of the infra-red system, each emitter shall have a minimum coverage area of 3800 square feet.  The receiver for either system shall conform to the following:

The unit must be a small, wearable unit capable of being clipped to a pocket or worn around the neck with an adjustable lavaliere cord.

The unit must be battery powered with either rechargeable (Ni-Cad) or disposable (alkaline) batteries.

The receiver design should allow rechargeable batteries to be charged without removal from the receiver.

The receiver must have a caution note inside the battery compartment warning the user not to recharge disposable (alkaline) batteries.

The receiver must be capable of operating at least 30 hours.

The receiver must have a charging indicator light.

The receiver must have an on-off switch/volume control, which adjusts the output level as required by the listener.

The receiver must have an output jack, which accepts both earphone and T-coil type accessories.

  - Microphones The system shall incorporate four (4) types of microphones to accommodate requirements for placement and operation (such as mix-minus and muting).

          - The first type (type 1) of microphone shall be a boundary type microphone for use on the counsel tables and shall be equipped with a mute switch with a visual indicator of such.  The microphone shall meet the following specifications: The microphone shall be a fixed-charge back plate permantently polarized condenser with a unidirectional polar pattern designed for use in surface mount boundary applications.  The frequency response shall be 30Hz to 16kHz (nom).  The maximum input sound level shall be 130dB SPL, 1kHz @ 1% T.H.D.  Typical dynamic range shall be 104dB, 1 kHz @ max SPL.  The microphone shall operate from an external 11V to 52 V DC, 4mA  phantom power source.  Output shall be low impedance (200 Ohm nom.) balanced.  The microphone shall have a touch-sensitive switch that may be set for any of three switching modes: on/off, momentary on, momentary off.  The microphone shall include a switch for low-frequency roll-off.  The microphone shall be provided with a minimum 25 ft. cable with the appropriate connectors installed.  The microphone shall be housed in a rugged perforated steel case and shall have a low gloss black finish.  Audio Technica U891R

          - The second type (type 2) of microphone shall be a gooseneck type microphone. The microphone shall be a fixed-charge condenser with a frequency response of 30 Hz to 20,000 Hz and a cardioid polar pattern with uniform 120° angle of acceptance. It shall be capable of accepting optional interchangeable elements for additional polar patterns.  It shall operate from an external 11V to 52V DC phantom power source. It shall be capable of handling sound input levels up to 138 dB with a dynamic range of 109 dB.  Nominal open-circuit output voltage shall be 10.0 mV at 1 kHz, 1 Pascal. Output shall be low impedance balanced (250 ohms).  The microphone shall be a small-diameter alternating gooseneck design with an integral adjustable-length section. The entire mechanical assembly shall operate smoothly and quietly for positioning.  The microphone shall have a self-contained power module with an XLRM-type connector at the base for direct connection to a mating XLRF-type panel jack or cable connector. The power module shall include a recessed switch for low-frequency roll-off. A shock mount shall be supplied for mounting in a solid surface. An included mounting adapter shall permit attaching the microphone directly to a standard 5/8"-27 thread. A snap-on foam windscreen shall also be included.  The microphone’s overall length shall be adjustable from 13.98" (355.0 mm) to 16.73" (425.0 mm). Head diameter shall be 0.33" (8.4 mm). The microphone weight shall be 4.4 oz (125 grams). Finish shall be low-reflectance black.  Audio Technica ES915C

          - The third type (type 3) of microphone shall be identical to the Type 2 microphone described above with the addition of mute switches for use on the council table.  Audio Technica ES917S/C

          - The fourth type (type 4) of microphone shall be a wireless (UHF) type.  The transmitter shall operate in the range of 581.200 to 781.775 MHz (nominally) and shall be either a handheld type or a lavalier type that can be easily attached to clothing.  The transmitter shall have a single power and mute switch and shall have a frequency response of 20 Hz to  15kHz, nominally.  The transmitter shall be capable of being powered by either a single 9V DC battery or multiple AA sized batteries.  The rated RF output of the transmitter shall be 50mW, maximum.  They shall be a “diversity” type incorporating dual antennas to achieve the highest  performance possible.  The receiver shall incorporate either XLR type or ¼” type connections for interfacing with the automixer. The receiver shall include a rack mount kit to allow placement within the equipment rack.   In the case where multiple wireless units may be required, the equipment supplier shall ensure that the units chosen shall not inferfere with each other or similar systems that may be in use nearby.  Audio-Technica 2000 Series

  - Speakers The system shall incorporate two (2) types of speakers to accommodate requirements for site restrictions and system operation.

The first type (type 1) of speaker shall be a ceiling type capable of constant voltage operation via a factory supplied, multi-tap, isolation transformer.  The speaker design may be either a “drop-in” or mountable type depending on site requirements.  In the case of a mountable type being used, tile bridges shall be required unless the installation method requires a retro fit into a finished ceiling surface.  In the case of a retro fit the appropriate manufacture’s mounting kit shall be used.  Back-boxes shall be used where required for plenum type ceilings.  System shall include a high performance 8" coaxial loudspeaker, ported bass reflex enclosure and press-fit grille for conventional ceiling installation.  Frequency response for the system shall be 55Hz to 20kHz (±5dB).  Sensitivity shall be 90dB average.  Loudspeaker shall be compromised of an 8" coaxial cone type driver.  Cone shall be constructed of polypropylene with a butyl rubber surround.  The19mm tweeter shall be constructed of PEI Magnet shall be a minimum of 21oz. (595g) and the voice coil diameter shall be 1" (25mm).  Transformer shall be (70.7/100V) volt type with 1.9, 3.8, 7.5, 15, 30 and 60 watt primary taps (@70.7V) with a front mounted tap selector switch. This tap selector switch shall also include a transformer bypass setting for instances where the 8 ohm FAP82T driver is to be direct coupled with a low impedance amplifier.  Enclosure shall be a deep drawn steel enclosure design acoustically treated with fiberglass lining. Internal volume shall be 937 cubic inches.  To facilitate connection in conduit systems, enclosure shall be equipped with an access panel covering a recessed terminal cup. This cover shall provide a combination in (19mm inside diameter) / 1" (25mm inside diameter) knock-out on the side access and a top access compression fitting/strain relief to facilitate flexible conduit up to 22mm outside diameter or 1 in (25mm inside diameter) conduit when the compression fitting is removed.  External wiring shall be accomplished via a removable lockable wiring connector with screw-down terminals to provide both secure wire termination and pre-wiring capability before loudspeaker installation. The 4 pole locking connector shall facilitate in/out connections and shall be located in the recessed area behind the conduit access panel. A 1⁄4"–20 drop forged eyebolt factory installed into a insert on top of enclosure for additional suspension point when used in drop tile ceilings or for pendant mounting.  Construction of enclosure shall be a minimum of 18 gauge deep drawn metal finished in white epoxy.  The system shall include a support backing plate to reinforce the ceiling material and tile support rails for use on either 2' x 4' (609mm x 1219mm) or 2' x 2' (609 x 609 mm) suspended ceiling tiles. This assembly can all be installed from beneath the ceiling tile.  Overall front face diameter shall not exceed 15.75" (400mm); overall height shall not exceed 11.5" (292mm), 12.75" (324mm) including factory-installed eyebolt.  Grilles shall be press-fit, manufactured from 24 gauge perforated steel mesh and finished in white epoxy. Round grille shall be 13.67” (347mm) diameter.  The loudspeaker shall weigh no more than 13.9 lbs (6.3kg)

Atlas Sound FAP82T

          - The second type (type 2) of speaker shall be a surface mount type capable of constant voltage operation via a factory supplied, multi-tap, isolation transformer.  Assemblies shall consist of two-way woofer and tweeter within environment-resistant housings. Enclosure shall be constructed of paintable UV-resistant, talc impregnated, polypropylene injection molded plastic finished in black or white. Each unit shall include a stamped powder coated aluminum grille and removable C-shaped mounting bracket. All hardware inserts shall be brass and threaded 1⁄4"-20. The 16-watt RMS system shall have a 4" (102mm) woofer, constructed of reinforced polypropylene, and a 1" (25mm) Ferro fluid tweeter. The dividing network crossover frequency shall be 4000 Hz. The dividing network shall include protection circuits for the high-frequency component. Each unit shall include an internally mounted 16 watt 70.7V/100V line matching transformer for use in distributed sound applications. Wattage taps shall be screwdriver selectable via a sealed switch located near the input section. Wattage taps shall be 2W, 4W, 8W, 16W @ 70.7V. The loudspeaker system shall meet the following performance criteria: Power handling, 30 watts RMS. Frequency response, +/- 5 dB from 115 Hz to 16 kHz. Sensitivity shall be 88dB SPL at one watt, 85 Hz to 16 kHz measured at a distance of one meter on axis. Impedance shall be 8 ohms nominal. Input connectors shall include a two-pole barrier strip capable of accepting up to (2) #14AWG cable. A cover with wire exit hole shall be provided to protect the input connectors. The loudspeaker system shall exhibit no performance or cosmetic degradation after a 100 - hour salt spray test per ASTM B117 The unit shall be 7"(178mm) high, 5" (127mm) wide, 5-15/16" (151mm) deep and weigh 1.5lbs (.68kg).  Atlas Sound SM42

  - Public Recording The system shall provide the press with a means of connecting equipment for the purpose of public recording.  The interface shall be wall mountable, shall be transformer coupled to the distributed speaker system, shall provide 100 dB of isolation, shall have a front panel accessible level trim adjustment, shall provide both RCA and ¼” TRS outputs, and shall provide output levels of –10dBv and –50dBv for line and mic out respectively.  Communications Company, Inc. RP-3030

  - Equipment Rack An Equipment Rack shall be provided to house all the electronic equipment associated with the system (excluding microphones and speakers).  The amplifier shall be provided with at least 1 blank vented panel above and below its relative position within the rack housing.  Blank panels shall be provided as required to ensure all available panel space is covered.  Middle Atlantic ERK Series

## 2.4 WIRE AND CABLE:

All cabling required for interconnecting the system shall be as follows and shall be plenum rated where required:

- Conductors:  Size speaker circuit conductors from racks to loudspeaker outlets not smaller than 16 gage and conductors from microphone receptacles to mixer not smaller than 24 gage.  Use jacketed, stranded, twisted-pair untinned solid copper conductors with a drain wire.

- Insulation for Wire in Conduit:  Thermoplastic not less than 1/32-inch thick.  The microphone and speaker cable shall be installed in metallic conduit.

- Shielding: 34-gage tinned soft copper strands formed into a braid or approved equivalent foil type.  Shielding coverage on the conductors not less than 60 percent.

- Microphone Cables:  Neoprene jacketed not less than 2/64 inch thick over shield with filled interstices.

System Contractor shall label each field cable that enters the head end control.  This includes all microphone cables, all loudspeakers cables, and any other cables entering the control.  The Systems contractor shall prepare a schedule of the field cable numbers with the application and location of the field equipment that the specific cable represents.  This information shall be permanently mounted in the head end equipment and become a required submittal to be included in the final Owner’s Manual.

# Part 3 – EXECUTION

## 3.1 EXAMINATION

- The Owner or the owner’s representative shall examine conditions, with the Installer present, for compliance with requirements and other conditions affecting the performance of the Microphone Control and Sound Reinforcement work.

- Do not proceed until unsatisfactory conditions have been corrected.

## 3.2 INSTALLATION

- General:  Install system in accordance with NFPA 70 and other applicable codes.  Install equipment in accordance with manufacturer’s written instructions.

- Wiring Methods:  Install wiring in a professional manner.  All microphone and speaker cable shall be identified, bundled and secured.

- Impedance and Level Matching:  Carefully match input and output impedances and signal levels at signal interfaces.  Provide matching networks where required.

- Control Circuit Wiring:  Install control circuits in accordance with NFPA 70 and as indicated.  Provide number of conductors as recommended by system manufacturer to provide control functions indicated or specified.

- Splices, Taps, and Terminations:  Make splices, taps, and terminations on numbered terminal strips in the head end equipment enclosures.  Do Not splice the cable between the microphones and the equipment enclosure.

- Identification of Conductors and Cables:  Use color coding of conductors and apply wire and cable marking tape to designate wires and cables so all media are identified in coordination with system wiring diagrams and other requirements as specified herein.

- Repairs:  Wherever walls, ceilings, floors, or other building finishes are cut for installation, repair, restore, and refinish to original appearance.

## 3.3 GROUNDING

- Provide equipment grounding connections for Microphone Control and Sound Reinforcement system as recommended by manufacturer.  Tighten connections to comply with tightening torques specified in UL Standard 486A to assure permanent and effective grounds.

- Ground equipment, conductor, and cable shields to eliminate shock hazard and to minimize to the greatest extent possible, ground loops, common mode returns, noise pickup, cross talk, and other impairments.  System Contractor to furnish and install dedicated ground for the Microphone Control and Sound Reinforcement system.

- To avoid system noise, data errors, safety hazards, and equipment damage, all devices and cabling shall be installed using a consistent grounding scheme.  All devices shall be grounded and all ground conductors shall follow a star topology.  Each equipment rack and piece of signal bearing equipment shall be connected such that there is never more than a single path to ground.  Ground conductors refered to in this section shall be 10AWG insulated solid copper cable.  Grounding conductors shall be terminated using a closed ring lug, of proper size for each application, which shall be connected to system electronic components and the equipment rack master bus using nuts, bolts, and lock washers.  Under no circumstances shall an AC neutral conductor be used to ground equipment.

## 3.4 FIELD QUALITY CONTROL

- Manufacturer’s Field Services:  Provide services of a factory authorized service representative to layout, plan and supervise the field assembly and connection of components and the pre-testing, testing, and adjustment of the system.

- Testing:  Upon completing installation of the system, align, adjust, and balance the system and perform complete testing.  Determine the conformance of the system to the requirements of the Drawings and Specifications.  Correct deficiencies observed.  Replace malfunctioning or damaged items with new, and retest until material satisfactory performance and conditions are achieved.

- Operational Test:  Perform an operational system test to verify conformance of system to the Specifications.  Observe sound reproduction for proper volume levels and freedom from noise.  This operational test shall be observed by the Owner or Owner’s Representative.  The Contractor shall give a two-week notice that the system is ready for this test to all parties involved.  At this time the system shall be evaluated and given final approval that it meets these specifications, System Contractor shall remove and replace equipment, reinstall, and connect the system at no cost until such time as the system meets these specifications.  Final approval of this system equipment will be given to the System’s Contractor in writing following the successful Operational Test and Inspection of the system within one week.

## COMMISSIONING

- Train Owner’s maintenance personnel in the procedures and schedules involved in operating, troubleshooting, servicing, and preventative maintenance of the system.  Provide a minimum of three hours training.  This training should ideally be conducted at the same time as the operational test.

- Schedule training with Owner with at least 7 days advance notice.  Training shall be conducted between the hours of 9:00 AM and 3:00 PM Monday through Friday

- Occupancy Adjustments:  When requested by the Owner within one year of date of Substantial Completion, provide one additional on-site visit to assist in adjusting sound levels, resetting matching transformer taps, and adjusting controls to suit actual occupied conditions.